Nice — below is a compact, practical catalogue of mmWave / radar sensor options that can give you the kind of data your AI team needs (range-Doppler arrays and/or point-clouds / raw-ish radar outputs), with current list/pricing references and an actionable recommendation for your $375 budget.

I grouped the sensors by what they actually provide (true raw I/Q / RD arrays / point clouds vs. range profiles or processed outputs), noted typical prices (vendor listings at time of writing), and gave quick pros/cons so you can decide fast.

## Quick summary / short recommendation

- If you need true range-Doppler arrays and point-clouds (best for custom ML): go TI IWR6843ISK or IWR1443 / AWR1843 evaluation kits — they expose point clouds / range-Doppler and have mature SDKs. Downside: they are expensive (~$200–$370 each) and will eat most of your $375 budget. (Texas Instruments, Mouser Electronics, DigiKey)

- Cheaper, practical alternative for prototyping / some raw data: Acconeer A111 (SparkFun / breakouts ~$50–$60) or Acconeer modules — give range profiles and are low-cost; good for early experiments but limited in Doppler/point-cloud richness compared to TI. (SparkFun Electronics, DigiKey)

- Very low-cost / flexible FMCW modules (confirm raw export before buying): Seeed / other 24 GHz FMCW modules (MR24HPC1 / MR24 variants). They are inexpensive but you must confirm they can stream range-Doppler or raw sweeps — vendor docs vary. (in.element14.com, Robu)

## Candidate sensors (range-Doppler / point-cloud capable first) — table

| Sensor / Dev Kit | What it can output (relevant to ML) | Typical price (approx.) | Pros | Cons | Source |
| --- | --- | --- | --- | --- | --- |
| TI IWR6843ISK (eval kit) | Raw access, range-Doppler, point-cloud, SDK (mmWave Studio / mmWave-SDK). Ideal for building RD arrays & point-clouds for ML. | ~$200–$350 (vendor listings vary; evaluation kit). (Texas Instruments, Octopart) | Full raw data, mature SDK and examples, USB-powered eval board, proven in research. | Pricier; steeper learning curve; may consume most of project budget. | (Texas Instruments, Mouser Electronics) |
| TI IWR1443BOOST / AWR1843BOOST | Evaluation kits that expose point-clouds and range/Doppler outputs and come with TI SDK support. | ~$350–$370 each (typical distributor price). (DigiKey) | Excellent raw data access and SDK support for range-Doppler and clustering → point clouds. | Expensive for a student budget; hardware/SDK complexity. | (DigiKey, Mouser Electronics) |
| Acconeer A111 (Pulsed PCR) + SparkFun / breakout | Range profiles (very accurate range data), can do motion detection; some Acconeer devices support multi-frame processing (can be used for Doppler-ish features). Not the same full RD array as TI but useful for ML on range/time signatures. | $50–$60 (SparkFun / retailers). Bulk chip prices much lower; breakout boards cost more. (SparkFun Electronics, DigiKey) | Low cost, low power, easy to prototype, good documentation. | Limited Doppler resolution / point-cloud richness compared to TI 60GHz mmWave chips. | (SparkFun Electronics, Acconeer) |
| Novelda / XeThru (X4M300 / X4M200) | High sensitivity presence & respiration; vendor tools expose processed waveforms and some raw telemetry; great for micro-motion and breathing detection. | Developer kits / evaluation modules vary (examples: dev kit pricing and module pricing vary; not always cheap). Historically dev kits ranged higher; contact vendor. (NOVELDA, Fierce Electronics) | Very sensitive to micro-movement (breath, heartbeat); excellent for human presence/time series. | May not expose full I/Q sweeps or RD arrays in the same way TI does; costs and availability vary. | (NOVELDA, Acconeer) |
| Seeed / MR24HPC1 / MR24 series (24 GHz FMCW modules) | Some are FMCW modules that can output configurable range data (range profiles) and visual debugging tools; vendor pages claim configurability — confirm raw export before buying. | $20–$80 depending on SKU and seller. (Seeed/ distributors have low prices). (in.element14.com, Robu) | Cheap, community support, easy integration, low cost leaves room in budget. | Many SKUs only export processed presence/motion; raw RD export often not guaranteed — must confirm. | (in.element14.com, diycomponents.in) |

## Price notes & citations

- TI eval kits (IWR1443 / AWR1843) are typically listed near $350 on distributor pages (Mouser, DigiKey). That matches current product pages. (DigiKey, Mouser Electronics)

- IWR6843ISK eval kits have varied prices; octopart/DigiKey/Mouser show listings (~$200–$300). Confirm vendor stock. (Texas Instruments, Octopart)

- Acconeer A111 breakout boards show up on SparkFun / Pimoroni / distributors around $50–$65. Good low-cost option to experiment with range/time data. (SparkFun Electronics, shop.pimoroni.com)

- Seeed / MR24HPC1 and other 24 GHz modules are inexpensive on Seeed / Element14 / reseller pages — but verify raw RD capability before purchasing. (in.element14.com, Robu)

(These vendor prices change frequently — check the product pages before ordering. I cited the pages above so you can click through.)

## Budget guidance for your $375 total

- TI kits (best data) consume most of budget. Example: buying one TI IWR1443BOOST/AWR1843BOOST at $350 leaves only $25 for Pi/ESP32/accessories — not realistic. (DigiKey, Mouser Electronics)

- Hybrid/cheap route that fits the budget:

  - Get an Acconeer A111 breakout ($60) + ESP32 ($10) + Pi Zero 2 W (~$15) or low-cost Pi + accessories — fits under $375 and lets you prototype real range/time modeling. (SparkFun Electronics, DigiKey)

  - If the data team strictly requires RD arrays / point-clouds, plan to request extra budget or borrow a TI eval board from a lab — those are the realistic options for full RD arrays.

## Next steps (recommended, in order)

- Decide exactly which radar data you need: (A) full I/Q sweeps, (B) range-Doppler arrays, or (C) range profiles/time series. TI kits = (A)/(B); Acconeer = (C) mainly.

- If RD arrays / point clouds are required: petition professor for extra funds or request access to a lab TI eval board (IWR6843 / IWR1443 / AWR1843). TI boards are the practical choice for serious ML work. (Texas Instruments, Mouser Electronics)

- If you must stay within $375 now: buy an Acconeer A111 breakout (or Seeed MR24 if vendor confirms raw range frames) and prototype ML on range/time features — later pivot to TI for richer data only if necessary. (SparkFun Electronics, in.element14.com)

- Contact vendors (quick script I can draft) to confirm “I can stream raw I/Q or range-Doppler arrays over USB/UART/ethernet — is that supported?” (do this before purchasing any cheap module).

## Would you like me to:

- Draft the vendor question message you should send to Seeed / DFRobot / TI to confirm raw data access? (I can make one that’s copy-paste ready.)

- Build a shopping list for the low-cost Acconeer-based prototype (links & approximate totals to fit $375)?

- Draft a short justification statement to your professor requesting access to a TI eval board for the ML/RAW-data requirement?