RADIAN - RADar-based Intelligent Activity moNitoring

Matthew Blackwell

Mitch Hoffman

Wyatt Johnson

Interface Control Document

Interface Control Document

for

RADIAN

Team 60

Approved by:

_____________________________

Matthew Blackwell	Date

_____________________________

Prof. Jang	Date

_____________________________

Roman Venegas      	Date

Change Record

| Rev. | Date | Originator | Approvals | Description |
| --- | --- | --- | --- | --- |
| 1 | 10/04/2025 | Matthew Blackwell |  | Draft Release |
| 2 | 12/05/2025 | Wyatt Johnson |  | Document formatting and updated Electrical Power System |

# Table of Contents

Table of Contents	3

List of Tables	5

List of Figures	6

1. Overview	7

2. References and Definitions	8

2.1. References	8

2.2. Definitions	8

3. Physical Interface	9

3.1.Weight	9

3.2. Dimensions	9

3.2.1. Dimensions of Data Subsystem	9

3.2.2. Dimensions of Physical Subsystem	9

3.2.3. Dimensions of Wireless Subsystem	9

3.3. Mounting	9

3.3.1. Height	9

3.3.2. Angle	9

3.3.3. Locations	10

4. Thermal Interface	11

5. Electrical Interface	12

5.1. Primary Input Power	12

5.1.1 Electrical	12

5.1.2 Connector	12

5.1.3 Wiring	12

5.2. Charging Interface	12

5.2.1 Input	12

5.2.2 Connectors	12

5.3. Grounding and ESD	13

5.3.1 Ground	13

5.3.2 ESD	13

5.4. User Control Interface	13

6. Communications / Device Interface Protocols	14

6.1. Wi-Fi Communication	14

6.2. Cellular Data	14

6.3. Internet Protocols	14

6.4. UART Serial Interface	14

7. Mobile Application Interface	15

7.1 Communication Flow	15

7.2 User Interface	15

7.3 Platform Compatibility	15

7.4 Notification Delivery	15

7.5 Performance Requirements	15

# List of Tables

Table 1: Data Subsystem Dimensions                                           			         8

Table 2: Physical Subsystem Dimensions		     	                                                      8

Table 3: Wireless Subsystem Dimensions				                                           8

# List of Figures

Figure 1: Electrical Power System							                    9

# 1. Overview

This Interface Control Document (ICD) will cover the inputs, outputs, controls, and connections between the Physical, Data, and Wireless subsystems of the RADIAN system. This document will be continuously reviewed and updated as the RADIAN group continues to build, test, and integrate each subsystem. The ICD will include descriptions of each component being used in the system, how they interact with one another, and what data is output from each. The electrical connections and wireless interfaces, and protocols will be described in detail to ensure correct integration for success as described in the Concept of Operations (ConOps).

# 2. References and Definitions

## 2.1. References

IEEE 802.11

IEEE Standard for Information Technology – Telecommunications and Information Exchange between Systems

Jan. 23, 1996

## 2.2. Definitions

3GPP			3rd Generation Partnership Project

API				Application Programming Interface

A				Ampere

AI				Artificial Intelligence

CPU			Central Processing Unit

ConOps			Concept of Operations

g 				Grams

GUI				Graphical User Interface

FSR			Functional System Requirements

HTTP			Hypertext Transfer Protocol

ICD				Interface Control Document

IEEE			Institute of Electrical and Electronics Engineers

ML				Machine Learning

mm 			Millimeter

REST		           Representational State Transfer

# 3. Physical Interface

## 3.1.Weight

The total weight of the RADIAN system, not including the Android device, will be less than or equal to 0.8 kilograms. This weight requirement enables the system to be properly mounted on the chosen room's wall without requiring heavy-duty hardware and support. Given that each subsystem has only a few components, the weight should not exceed the RADIAN group’s given design.

## 3.2. Dimensions

### 3.2.1. Dimensions of Data Subsystem

| Component | Length | Width | Height | Weight |
| --- | --- | --- | --- | --- |
| Raspberry Pi 3 | 85.6 mm | 56.5 mm | 17 mm | 41.2 g |

Table 1: Data Subsystem Dimensions

### 3.2.2. Dimensions of Physical Subsystem

| Component | Length | Width | Height | Weight |
| --- | --- | --- | --- | --- |
| 6.8Ah Power Board | 135.38 mm | 90.46 mm | <2 mm | <90 g |
| Radome | 190 mm | 120 mm | 55 mm | <230 g |

Table 2: Physical Subsystem Dimensions

### 3.2.3. Dimensions of Wireless Subsystem

| Component | Length | Width | Height | Weight |
| --- | --- | --- | --- | --- |
| TI AWR6843ISK | 103 mm | 57 mm | 16 mm | 91 g |

Table 3: Wireless Subsystem Dimensions

## 3.3. Mounting

### 3.3.1. Height

The RADIAN system will need to be mounted at a minimum of 2 meters off the ground. This allows the system to properly scan the room, have good line-of-sight, and detect if a fall has occurred.

### 3.3.2. Angle

The RADIAN system must be mounted to the wall at a 45-degree angle to allow for the maximum amount of area to be scanned by the radar. Having the system parallel to the ground may prevent certain areas of the room from being scanned and, therefore, prevent proper detection.

### 3.3.3. Locations

The RADIAN system can be located in any common room of a home or building, including but not limited to living rooms, dining rooms, bedrooms, and kitchens. The RADIAN system is not rated for humidity or water exposure, so it cannot be used in the bathroom.

# 4. Thermal Interface

With the Raspberry Pi 3 and TI AWR6843 chip being enclosed in a 3D printed structure, the added thermal heat could cause reliability issues as the RADIAN system runs. Because the system runs continuously, this further increases the chance for potential failures due to overheating. To mitigate these risks, a small heatsink will be added to the Raspberry Pi 3 to ensure that it remains cool as it handles the data and accurately predicts if a fall occurred. A small 30 mm fan will also be installed to provide airflow across both the Raspberry Pi 3 and the AWR6843 chip. These solutions will allow the RADIAN system to run reliably and efficiently.

# 5. Electrical Interface

Figure 1: Electrical Power System

## 5.1. Primary Input Power

### 5.1.1 Electrical

The primary input power of the system will be two 18650 Li-ion cells in parallel, 3.7 V nominal each, for a 7 Ah total capacity. The battery voltage will be stepped up to 5V using a boost converter with a maximum continuous current of 2-3 A.

### 5.1.2 Connector

4-pin locking connector with two positive terminals and two negative terminals for the Raspberry Pi 3 and TI AWR6843.

Pinout (example): 1 = BAT+, 2 = BAT+, 3 = GND, 4 = GND.

### 5.1.3 Wiring

18–20 AWG wiring between the power board and the Raspberry Pi 3/ TI AWR6843. (Target ≤2% drop on the 5 V rail.)

## 5.2. Charging Interface

### 5.2.1 Input

The battery system will be charged using a 5V, 1A wall adapter barrel jack. Charging at a low current will keep the batteries from overheating and ensure safety.

### 5.2.2 Connectors

The battery system will receive its power through a 5V barrel jack connected to a wall adapter.

## 5.3. Grounding and ESD

### 5.3.1 Ground

Single ground plane located on the power PCB.

### 5.3.2 ESD

TVS diodes on the barrel jack and on external connectors.

## 5.4. User Control Interface

The RADIAN system will come with a custom-designed phone application where the caregiver will be able to receive push notifications that a fall has been detected. The app may include a simple battery monitoring system to alert the user when the batteries need to be recharged.

# 6. Communications / Device Interface Protocols

## 6.1. Wi-Fi Communication

The Raspberry Pi 3 will use the IEEE 802.11b/g/n Wi-Fi standards through its onboard communication chip. These standards apply to Wi-Fi 1, Wi-Fi 3, and Wi-Fi 4, each offering new advancements in performance.

## 6.2. Cellular Data

The Gotify server will communicate with the phone application by 5G cellular data, given by the 3GPP standards for 5G wireless communication.

## 6.3. Internet Protocols

The Raspberry Pi 3 will post the alert message through the standard HTTP REST API protocol, allowing the message to be communicated from the Raspberry Pi 3 to the Gotify server.

## 6.4. UART Serial Interface

A UART serial interface will connect the Raspberry Pi 3 to the TI AWR6843ISK. Data shall be exchanged using packets containing radar object data. The Raspberry Pi shall use these packets and convert the radar output into the AI algorithm, which is then sent to the Gotify server.

# 7. Mobile Application Interface

## 7.1 Communication Flow

The Raspberry Pi 3 shall send event data to the Gotify server using an HTTP POST request. The Gotify server will then relay the data to the mobile app via WebSocket connection to receive messages in real time.

## 7.2 User Interface

The app shall display incoming alerts and system messages, battery life, and previous alerts, if any, in a custom-designed user interface.

## 7.3 Platform Compatibility

The app will be developed using Flutter for the user interface and Dart for the programming language. This gives the ability for the app to be run on Android devices.

## 7.4 Notification Delivery

The app shall generate a push notification when it has received an alert message from the Gotify server. The notifications will be displayed using the notification system of the Android device, to ensure the notifications are visible when the app is not open.

## 7.5 Performance Requirements

The alerts shall be delivered to the app within 5 seconds of the Raspberry Pi sending the alert to the Gotify server, under normal network conditions.