Stage: Pre-Power
Test ID: PP-02
Test name: Resistance-to-ground check
Objective: Detect gross shorts
Equipment: DMM
Procedure: Measure resistance between +BAT, +5V to GND; and between each other
Pass criteria: All readings > 100 Ohm (capacitive charging may show rising readings)
Readings:

- R(+BAT to GND): ___23.4kOhm___

- R(+5V to GND): ___5MOhm___

- R(+BAT to +5V): ___0.6MOhm___

Notes: __________________________________________
Result (P/F): ___P___ Date: ___11/10/2025___